Article 2274 of comp.sys.atari.st:

>From: kleef@ark.cs.vu.nl (Patrick van Kleef)

Subject: Re: Audio *improvement* on ST

Keywords: audio - st

Date: 6 Apr 87 23:46:07 GMT

Organization: VU Informatica, Amsterdam

After adding a video-recorder and a compactdisc player to my

audio-collection, things got too crowded for my old amplifier. So

I bought a new one with more facilities. After putting the old amplifier

away in some deep, dark and dusty cupboard, I figured it might be nice

to use it to improve the poor sound my SM124 monitor provides.

Fortunately (and probably by accident :) _every_ ST has a normal,

standard audio output signal that can be hooked up to any piece of audio

equipment. Thus you can use an old amplifier and a small (or

-preferably- large) speaker to reflect all the *poing*, *bang*, *arrgh*

and whatever other sounds programs generate.

The connection can be made WITHOUT voiding your warranty

What you need is two pieces of wire, both with a spring-clamp (hope

that is the right word :) on one and, and bare wire at the other. If you

open the lid you can see the extremely_multi video connector. Pins

run from the main PCB into the connector.

From one of these pins, we're gonna get our audio signal. As you can

read in your Atari manual, pin 1 is used for this signal. When you look

into the connector from the _outside_, it's located at the top rightmost

position. So from the inside the top leftmost position.

1) Find pin one and attach the springclamp to it.

2) Use any position on the internal metal hood for the ground

connection.

3) Solder the 'pin 1 signal' wire the the _internal_ part of

the RCA connector.

4) Solder the ground wire to the external part of the RCA connector.

5) Use any standard RCA-plug to connect the audio signal to

your amplifier.

6) Admire the GREATLY improved sound! (after connecting a speaker,

of course - any car speaker suffides)

As you may have noticed, we haven't made any structural changes in the

ST.

If your computer decides to go into hibernation (RIP), you can safely

remove all attachments and bring the machine to your dealer. Since my

warranty has expired anyway, I made a more permanent arrangement.

I drilled a hole in the back of my 1040 ST (the place where that

modulator was intended to rest is ideal) and screwed in a female RCA

connector. In the same run I added an extra plug for possible future

video-output.

It sounds great!